ICS / OT / IoT / CPS

Cyber Security Lifecycle

Templates, Procedures & Specifications

Based on ISA 84.00.09 Edition 3 Lifecycle Methodology

Applicable to ALL Automation and Control System Types

Safety Systems, Primary Control, IoT, and Cyber-Physical Systems

Cross-Referenced Standards: ISA/IEC 62443, NIST SP 800-82, NIST SP 800-160,

MITRE ATT&CK for ICS, MITRE EMB3D, NERC CIP, IEC 61511, IEC 61508

[Organization Name]

[Date]

Document Version: [Version]

# Table of Contents

Table of Contents	2

1. Document Control	7

1.1 Revision History	7

1.2 Distribution List	7

1.3 Applicable System Types	7

1.4 Referenced Standards and Frameworks	8

2. Initial Cyber Security Assessment (ICSA)	10

2.1 System Inventory and Characterization	10

2.2 High Consequence Event Identification	10

2.3 Cyber Security Risk Matrix	10

2.4 Initial Vulnerability Assessment	11

3. Detailed Cyber Risk Assessment (DCRA)	12

3.1 Threat Actor Profiling	12

3.2 Attack Scenario Development	12

3.3 Security Countermeasure Gap Analysis	13

3.4 MITRE EMB3D Embedded Device Threat Analysis	13

4. Cyber Security Requirements Specification (CSRS)	15

4.1 Zone and Conduit Security Level Targets	15

4.2 Detailed Security Requirements	15

4.3 Security Requirements Traceability Matrix	16

5. Design and Engineering	17

5.1 Zone and Conduit Architecture Design	17

5.2 Security Control Design Specification	17

6. Implementation and Integration	19

6.1 Implementation Plan	19

6.2 Secure Configuration Baseline	19

6.3 Change Management Procedure	20

7. Verification and Validation (V&V)	21

7.1 Security Test Plan	21

7.2 Security Test Results	21

7.3 Penetration Test Report Summary	22

8. Operations and Maintenance	23

8.1 Security Monitoring Procedures	23

8.2 Patch Management Procedure	23

8.3 Incident Response Plan for ICS/OT	24

9. Modification, Retrofit, and Decommissioning	25

9.1 Security Assessment for Modifications	25

9.2 Decommissioning Security Checklist	25

10. Plan of Action and Milestones (POA&M)	27

11. Key Performance Indicators - Complete Annex L (ISA 84.00.09 Edition 3 Part 1)	28

L.1 Cyber Security Management System (CSMS) Performance	28

L.1.1: CSMS Policy Compliance Rate	28

L.1.2: Management Review Completion Rate	28

L.1.3: Cyber Security Resource Allocation Adequacy	29

L.1.4: Cyber Security Budget Execution Rate	29

L.1.5: Roles and Responsibilities Coverage	30

L.2 Risk Assessment Performance	30

L.2.1: ICSA Completion Rate	30

L.2.2: DCRA Completion Rate	31

L.2.3: Risk Assessment Currency Rate	32

L.2.4: Identified Risk Mitigation Rate	32

L.2.5: Residual Risk Acceptance Documentation Rate	33

L.2.6: HCE Coverage Rate	33

L.3 Security Architecture and Design	34

L.3.1: Zone and Conduit Documentation Completeness	34

L.3.2: Security Level Target (SL-T) Achievement Rate	34

L.3.3: Network Segmentation Verification Rate	35

L.3.4: Defense-in-Depth Layer Coverage	35

L.3.5: Security Architecture Review Currency	36

L.4 Threat Management	36

L.4.1: Threat Intelligence Integration Rate	36

L.4.2: Threat Assessment Update Frequency	37

L.4.3: Attack Surface Reduction Progress	37

L.4.4: MITRE ATT&CK for ICS Coverage Rate	38

L.5 Vulnerability Management	38

L.5.1: Vulnerability Identification Rate	38

L.5.2: Critical Vulnerability Remediation Rate	39

L.5.3: High Vulnerability Remediation Rate	39

L.5.4: Medium/Low Vulnerability Remediation Rate	40

L.5.5: Mean Time to Remediate (MTTR) - Critical	40

L.5.6: Mean Time to Remediate (MTTR) - All Severities	41

L.5.7: Patch Currency Rate	41

L.5.8: Compensating Control Coverage for Unpatched Systems	42

L.6 Incident Management	43

L.6.1: Mean Time to Detect (MTTD)	43

L.6.2: Mean Time to Respond (MTTR-I)	43

L.6.3: Mean Time to Recover (MTTR-R)	44

L.6.4: Incident Response Plan Test Completion Rate	44

L.6.5: Incident Recurrence Rate	45

L.6.6: Lessons Learned Implementation Rate	45

L.7 Compliance and Audit	46

L.7.1: Security Policy Compliance Rate	46

L.7.2: Audit Finding Closure Rate	46

L.7.3: Regulatory Compliance Achievement Rate	47

L.7.4: Configuration Baseline Compliance Rate	47

L.7.5: Access Control Compliance Rate	48

L.8 Training and Awareness	48

L.8.1: Security Training Completion Rate	48

L.8.2: Role-Based Training Coverage Rate	49

L.8.3: Security Awareness Assessment Score	49

L.8.4: Social Engineering Exercise Performance	50

L.9 Supply Chain Security	50

L.9.1: Vendor Security Assessment Completion Rate	50

L.9.2: Supply Chain Risk Monitoring Coverage	51

L.9.3: Third-Party Access Compliance Rate	51

L.9.4: Software Bill of Materials (SBOM) Coverage	52

L.10 Operational Effectiveness	53

L.10.1: Backup and Recovery Test Success Rate	53

L.10.2: Security-Related System Availability	53

L.10.3: Change Management Security Review Completion	54

L.10.4: POA&M Item Closure Rate	54

L.10.5: POA&M Item Aging (Average Days Open)	55

L.10.6: Security Monitoring Coverage Rate	55

L.10.7: Security Control Effectiveness Score	56

11.2 Annex L KPI Summary Tracking Table	57

11.3 KPI Reporting Template	62

12. Multi-Framework Compliance Mapping	63

Appendix A: Glossary and Abbreviations	64

# 1. Document Control

## 1.1 Revision History

| Version | Date | Author | Description | Approved By |
| --- | --- | --- | --- | --- |
| 1.0 | [Date] | [Author] | Initial release | [Approver] |

## 1.2 Distribution List

| Role | Name | Organization | Access Level |
| --- | --- | --- | --- |
| Process Engineer | [Name] | [Org] | Full |
| Safety Engineer | [Name] | [Org] | Full |
| Control System Engineer | [Name] | [Org] | Full |
| Security Engineer | [Name] | [Org] | Full |
| System Integrator | [Name] | [Org] | Restricted |
| Project Manager | [Name] | [Org] | Summary |
| QA/QC Lead | [Name] | [Org] | Full |
| Operational User | [Name] | [Org] | Relevant Sections |

## 1.3 Applicable System Types

This document applies to ALL the following automation and control system types. Users must identify and include all system types present in their facility/project scope:

| System Type | In Scope | Zone/Conduit | Notes |
| --- | --- | --- | --- |
| Safety Instrumented Systems (SIS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Basic Process Control Systems (BPCS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Distributed Control Systems (DCS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Programmable Logic Controllers (PLC) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Remote Terminal Units (RTU) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| SCADA Systems | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Human Machine Interfaces (HMI) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Historians and Data Servers | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Industrial IoT Gateways and Edge Devices | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Smart Sensors and Actuators | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Motor Control Centers (MCC) and VFDs | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Fire and Gas Systems (FGS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Burner Management Systems (BMS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Turbine Control Systems (TCS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Pipeline SCADA and Leak Detection | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Building Automation Systems (BAS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Energy Management Systems (EMS/DERMS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Robotic and Autonomous Systems | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Physical Access Control Systems (PACS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |
| Video Surveillance (CCTV/IVS) | [Yes/No/NA] | [Zone/Conduit ID] | [Notes] |

## 1.4 Referenced Standards and Frameworks

| Standard | Edition/Version | Applicable Sections |
| --- | --- | --- |
| ISA 84.00.09 | Edition 3 | All phases |
| ISA/IEC 62443-2-1 | 2024 | CSMS requirements |
| ISA/IEC 62443-3-3 | 2013 | System security requirements |
| ISA/IEC 62443-4-1 | 2018 | Secure development lifecycle |
| ISA/IEC 62443-4-2 | 2019 | Component security requirements |
| NIST SP 800-82 Rev 3 | 2023 | ICS security guide |
| NIST SP 800-160 Vol 1/2 | 2022 | Systems security engineering |
| MITRE ATT&CK for ICS | Latest | Threat modeling, attack paths |
| MITRE EMB3D | Latest | Embedded device threat model |
| IEC 61511 | 2016+A1:2017 | Functional safety (SIS) |
| IEC 61508 | 2010 | Functional safety (general) |
| NERC CIP | v6/v7 | Bulk electric system (if applicable) |

# 2. Initial Cyber Security Assessment (ICSA)

Purpose: Establish the baseline understanding of cyber risk exposure for automation and control systems. This phase applies equally to safety systems (SIS, BMS, FGS) and primary control systems (BPCS, DCS, PLC, SCADA, IoT devices).

## 2.1 System Inventory and Characterization

Complete the following inventory for every system, device, and component within the assessment boundary:

| Asset ID | System Type | Manufacturer/Model | Firmware/Software Version | Network Zone | Conduit ID | SIL/Criticality | Protocol(s) | Connectivity | Physical Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [ID] | [Type] | [Mfg/Model] | [Version] | [Zone] | [Conduit] | [Level] | [Protocols] | [Connected To] | [Location] |

## 2.2 High Consequence Event Identification

Identify potential high consequence events (HCEs) resulting from cyber compromise of each system type. Consider safety, environmental, operational, financial, and reputational impacts.

| HCE ID | Description | Affected System(s) | Consequence Category | Severity (1-5) | Likelihood Basis | Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| [HCE-001] | [Description] | [Systems] | Safety / Environmental / Operational / Financial / Reputational | [1-5] | [Basis] | [H/M/L] |

## 2.3 Cyber Security Risk Matrix

Risk classification matrix for cyber-related scenarios. Each identified HCE shall be mapped to this matrix.

|  | Negligible (1) | Minor (2) | Moderate (3) | Major (4) | Catastrophic (5) |
| --- | --- | --- | --- | --- | --- |
| Almost Certain (5) | Medium | High | Critical | Critical | Critical |
| Likely (4) | Low | Medium | High | Critical | Critical |
| Possible (3) | Low | Medium | Medium | High | Critical |
| Unlikely (2) | Low | Low | Medium | Medium | High |
| Rare (1) | Low | Low | Low | Medium | Medium |

## 2.4 Initial Vulnerability Assessment

| Vuln ID | Asset/System | Vulnerability Description | CVE/Advisory | CVSS Score | Exploitability | Compensating Controls | Risk After Controls |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [V-001] | [Asset] | [Description] | [CVE] | [Score] | [H/M/L] | [Controls] | [H/M/L] |

# 3. Detailed Cyber Risk Assessment (DCRA)

The DCRA builds on the ICSA with deeper analysis of attack scenarios, threat actors, and security countermeasure adequacy for all system types.

## 3.1 Threat Actor Profiling

| Threat Actor Category | Capability Level | Motivation | Relevant System Targets | Historical Precedent | Risk Weighting |
| --- | --- | --- | --- | --- | --- |
| Nation-State APT | Very High | Espionage / Disruption / Destruction | SIS, BPCS, DCS, SCADA, IoT | [Examples] | [Weight] |
| Cybercriminal (Ransomware) | High | Financial Gain | HMI, Historians, IT/OT Boundary | [Examples] | [Weight] |
| Hacktivist | Medium | Ideological / Public Attention | SCADA, HMI, Public-facing | [Examples] | [Weight] |
| Insider Threat | Variable | Disgruntlement / Financial | All systems with access | [Examples] | [Weight] |
| Supply Chain Compromise | High | Broad Access | PLC, RTU, IoT firmware, SI tools | [Examples] | [Weight] |
| Terrorist | Medium-High | Physical Destruction / Fear | SIS, BMS, FGS, Critical Infra | [Examples] | [Weight] |

## 3.2 Attack Scenario Development

For each high-risk HCE, develop detailed attack scenarios using MITRE ATT&CK for ICS tactics and techniques:

| Scenario ID | Target System | Attack Objective | Initial Access | Execution | Lateral Movement | Impact Technique | MITRE ATT&CK ICS ID(s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [AS-001] | [System] | [Objective] | [T0817/T0819/...] | [T0807/...] | [T0812/...] | [T0813/T0831/...] | [Technique IDs] |

## 3.3 Security Countermeasure Gap Analysis

Evaluate existing countermeasures against identified attack scenarios. Map to ISA/IEC 62443 foundational requirements and NIST SP 800-82 controls:

| Control Area | ISA 62443 FR | NIST 800-82 Control | Current State | Gap Description | Required Action | Priority | Owner |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Access Control | FR1 - IAC | AC-2, AC-3, AC-6 | [State] | [Gap] | [Action] | [H/M/L] | [Owner] |
| Use Control | FR2 - UC | AC-7, CM-7 | [State] | [Gap] | [Action] | [H/M/L] | [Owner] |
| System Integrity | FR3 - SI | SI-3, SI-7, CM-3 | [State] | [Gap] | [Action] | [H/M/L] | [Owner] |
| Data Confidentiality | FR4 - DC | SC-8, SC-12, SC-13 | [State] | [Gap] | [Action] | [H/M/L] | [Owner] |
| Restricted Data Flow | FR5 - RDF | SC-7, AC-4 | [State] | [Gap] | [Action] | [H/M/L] | [Owner] |
| Timely Response | FR6 - TRE | AU-6, IR-4, IR-5 | [State] | [Gap] | [Action] | [H/M/L] | [Owner] |
| Resource Availability | FR7 - RA | CP-7, CP-9, SC-5 | [State] | [Gap] | [Action] | [H/M/L] | [Owner] |

## 3.4 MITRE EMB3D Embedded Device Threat Analysis

For IoT devices, smart sensors, actuators, PLCs, RTUs, and embedded controllers, apply the MITRE EMB3D threat model:

| Device ID | Device Type | EMB3D Threat Category | Threat Property | Attack Surface | Mitigation | Status |
| --- | --- | --- | --- | --- | --- | --- |
| [D-001] | [Type] | Hardware / Firmware / Software / Network | [Property] | [Surface] | [Mitigation] | [Open/Closed] |

# 4. Cyber Security Requirements Specification (CSRS)

Define security requirements for each zone, conduit, system, and component. Requirements derive from the DCRA findings and map to ISA/IEC 62443 Security Levels (SL).

## 4.1 Zone and Conduit Security Level Targets

| Zone/Conduit ID | Name | Description | System Types Contained | SL-T (Target) | SL-C (Capability) | SL-A (Achieved) | Gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [Z-001] | [Name] | [Desc] | [SIS, BPCS, etc.] | [SL 1-4] | [SL 1-4] | [SL 1-4] | [Y/N] |

## 4.2 Detailed Security Requirements

| Req ID | Requirement Description | Source Standard | Applicable Systems | SL | Verification Method | Owner | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [SR-001] | User identification and authentication for all access points | 62443-3-3 SR 1.1 | All | SL 2+ | Test / Inspection | [Owner] | [Status] |
| [SR-002] | Enforce least privilege for all user and service accounts | 62443-3-3 SR 1.3 | All | SL 1+ | Configuration audit | [Owner] | [Status] |
| [SR-003] | Control system software integrity verification | 62443-3-3 SR 3.4 | PLC, RTU, SIS | SL 2+ | Hash validation test | [Owner] | [Status] |
| [SR-004] | Network segmentation between zones per architecture diagram | 62443-3-3 SR 5.1 | All | SL 2+ | Penetration test | [Owner] | [Status] |
| [SR-005] | Audit logging of security-relevant events | 62443-3-3 SR 6.1 | All | SL 1+ | Log review | [Owner] | [Status] |
| [SR-006] | System backup and recovery capability | 62443-3-3 SR 7.3 | All | SL 1+ | Recovery test | [Owner] | [Status] |
| [SR-007] | Embedded device firmware integrity per EMB3D mitigations | MITRE EMB3D | PLC, RTU, IoT, Sensors | SL 2+ | Firmware analysis | [Owner] | [Status] |

## 4.3 Security Requirements Traceability Matrix

Map every security requirement to its source threat/vulnerability, applicable ISA/IEC 62443 requirement, verification method, and responsible party:

| Req ID | Source Threat/HCE | 62443 Mapping | NIST 800-82 Mapping | MITRE ATT&CK/EMB3D | Test Case ID | V&V Status |
| --- | --- | --- | --- | --- | --- | --- |
| [SR-001] | [HCE-001, AS-001] | [SR 1.1] | [IA-2] | [T0859] | [TC-001] | [Pass/Fail/Pending] |

# 5. Design and Engineering

Translate security requirements into architecture, network, and system design. Apply defense-in-depth across all system types.

## 5.1 Zone and Conduit Architecture Design

Document the network architecture with zones, conduits, data flows, and trust boundaries. Include all system types identified in Section 1.3.

| Design Element | Description | Reference Drawing | Responsible Engineer |
| --- | --- | --- | --- |
| Zone Architecture Diagram | Logical zones with SL assignments per ISA 62443-3-2 | [Drawing #] | [Engineer] |
| Conduit Specifications | Protocols, ports, data flows, filtering rules per conduit | [Drawing #] | [Engineer] |
| Network Topology | Physical and logical network layout including firewalls, DMZs | [Drawing #] | [Engineer] |
| Data Flow Diagrams | All data flows crossing zone boundaries | [Drawing #] | [Engineer] |
| Wireless Architecture | WiFi, WirelessHART, ISA100.11a, LoRaWAN, cellular IoT | [Drawing #] | [Engineer] |
| Remote Access Architecture | VPN, jump hosts, MFA for remote connections | [Drawing #] | [Engineer] |
| Cloud/Edge Architecture | IoT cloud connections, edge computing, data lakes | [Drawing #] | [Engineer] |

## 5.2 Security Control Design Specification

| Control Category | Design Specification | Applicable System Types | Implementation Technology | Owner |
| --- | --- | --- | --- | --- |
| Access Control | Role-based access, MFA for engineering workstations, physical key locks for safety controllers | All | [Technology] | [Owner] |
| Network Security | Industrial firewalls, unidirectional gateways, VLAN segmentation, protocol-aware deep packet inspection | All networked | [Technology] | [Owner] |
| Endpoint Protection | Application whitelisting, USB device control, secure boot, firmware signing | HMI, EWS, PLC, RTU, IoT | [Technology] | [Owner] |
| Monitoring and Detection | ICS-aware intrusion detection, protocol anomaly detection, baseline monitoring | All networked | [Technology] | [Owner] |
| Backup and Recovery | Automated configuration backup, tested restoration procedures, offline backup storage | All | [Technology] | [Owner] |
| Physical Security | Cabinet locks, tamper detection, CCTV coverage of critical systems | SIS, BPCS, Network infra | [Technology] | [Owner] |

# 6. Implementation and Integration

## 6.1 Implementation Plan

| Phase | Activity | Systems Affected | Prerequisites | Duration | Responsible | Verification |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Network segmentation deployment | [Systems] | [Prerequisites] | [Duration] | [Responsible] | [Method] |
| 2 | Access control implementation | [Systems] | [Prerequisites] | [Duration] | [Responsible] | [Method] |
| 3 | Endpoint hardening | [Systems] | [Prerequisites] | [Duration] | [Responsible] | [Method] |
| 4 | Monitoring deployment | [Systems] | [Prerequisites] | [Duration] | [Responsible] | [Method] |
| 5 | Backup system configuration | [Systems] | [Prerequisites] | [Duration] | [Responsible] | [Method] |

## 6.2 Secure Configuration Baseline

Document the hardened configuration baseline for each system type. Reference CIS Benchmarks, DISA STIGs, and vendor hardening guides.

| System Type | Baseline Standard | Configuration Items | Hardening Guide Reference | Deviation Justification |
| --- | --- | --- | --- | --- |
| Windows HMI/EWS | CIS Windows Server / DISA STIG | [Items] | [Reference] | [Justification] |
| Linux-based Controllers | CIS Linux / DISA STIG | [Items] | [Reference] | [Justification] |
| PLC/RTU Firmware | Vendor hardening guide + EMB3D | [Items] | [Reference] | [Justification] |
| Network Switches/Firewalls | CIS Network / Vendor guide | [Items] | [Reference] | [Justification] |
| IoT Devices/Sensors | OWASP IoT / EMB3D | [Items] | [Reference] | [Justification] |

## 6.3 Change Management Procedure

| Step | Activity | Responsible Role | Required Documentation | Approval Authority |
| --- | --- | --- | --- | --- |
| 1 | Change request submission with security impact analysis | Requestor | Change request form, security impact assessment | [Authority] |
| 2 | Security review of proposed change | Security Engineer | Security review checklist, risk assessment update | [Authority] |
| 3 | Safety impact assessment (if applicable) | Safety Engineer | Safety impact analysis, SIL verification | [Authority] |
| 4 | Test in non-production environment | System Integrator | Test plan, test results | [Authority] |
| 5 | Implementation with rollback plan | Control Engineer | Implementation procedure, rollback procedure | [Authority] |
| 6 | Post-change verification and validation | QA/QC + Security | V&V report, updated baseline | [Authority] |

# 7. Verification and Validation (V&V)

Verify that all security requirements from Section 4 are correctly implemented and validate that the system meets its security objectives. Applies to ALL system types.

## 7.1 Security Test Plan

| Test ID | Test Objective | Target System(s) | Test Type | Test Procedure Reference | Pass Criteria | Prerequisites | Risk/Impact |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [TC-001] | Verify user authentication enforcement | [System] | Functional | [Procedure] | All unauthorized access attempts rejected | [Prerequisites] | Low |
| [TC-002] | Verify network segmentation effectiveness | [System] | Penetration Test | [Procedure] | No unauthorized zone traversal | [Prerequisites] | Medium |
| [TC-003] | Verify firmware integrity mechanisms | [System] | Security Analysis | [Procedure] | Unauthorized firmware rejected | [Prerequisites] | Low |
| [TC-004] | Validate backup and recovery procedures | [System] | Recovery Test | [Procedure] | System restored within RTO | [Prerequisites] | Medium |
| [TC-005] | Verify audit logging completeness | [System] | Inspection | [Procedure] | All security events captured | [Prerequisites] | Low |

## 7.2 Security Test Results

| Test ID | Date Executed | Tester | Result | Findings | Evidence Reference | Corrective Action |
| --- | --- | --- | --- | --- | --- | --- |
| [TC-001] | [Date] | [Tester] | Pass / Fail / Partial | [Findings] | [Evidence] | [Action if Fail] |

## 7.3 Penetration Test Report Summary

| Finding ID | Severity | Affected System | Attack Vector | MITRE ATT&CK ICS Technique | Description | Recommendation | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [PT-001] | Critical / High / Medium / Low / Info | [System] | [Vector] | [T-ID] | [Description] | [Recommendation] | [Open/Remediated] |

# 8. Operations and Maintenance

## 8.1 Security Monitoring Procedures

| Monitoring Area | Tool/Method | Frequency | Responsible | Escalation Procedure | Applicable Systems |
| --- | --- | --- | --- | --- | --- |
| Network traffic anomaly detection | [Tool] | Continuous | [Role] | [Procedure] | All networked systems |
| Endpoint integrity monitoring | [Tool] | Daily | [Role] | [Procedure] | HMI, EWS, Servers |
| Access log review | [Tool] | Weekly | [Role] | [Procedure] | All |
| Firmware version verification | [Tool] | Monthly | [Role] | [Procedure] | PLC, RTU, IoT |
| Vulnerability scanning (passive) | [Tool] | Quarterly | [Role] | [Procedure] | All reachable |
| Physical security inspection | Visual inspection | Weekly | [Role] | [Procedure] | SIS, BPCS cabinets |

## 8.2 Patch Management Procedure

| Step | Activity | Timeline | Responsible | Documentation Required |
| --- | --- | --- | --- | --- |
| 1 | Vendor patch notification received and logged | Within 24 hours | [Role] | Patch notification log |
| 2 | Security and safety impact assessment | Within 72 hours | Security + Safety Eng. | Impact assessment form |
| 3 | Lab/staging environment testing | Within 2 weeks (critical: 48h) | [Role] | Test results report |
| 4 | Change management approval | Before deployment | [Authority] | Approved change request |
| 5 | Deployment during maintenance window | Scheduled | [Role] | Deployment procedure, rollback plan |
| 6 | Post-deployment verification | Within 24 hours | QA/QC | Verification checklist |

## 8.3 Incident Response Plan for ICS/OT

| Phase | Actions | Responsible Roles | Communication Requirements | Documentation |
| --- | --- | --- | --- | --- |
| Detection | Identify anomaly via monitoring tools, operator observation, or alert | Operators, SOC, Security Eng. | Internal notification per escalation matrix | Detection log entry |
| Containment | Isolate affected systems, preserve evidence, maintain safety functions | Security Eng., Control Eng., Safety Eng. | Management notification, consider regulatory | Containment actions log |
| Eradication | Remove threat, patch vulnerabilities, restore from known-good backups | Security Eng., System Integrator | Status updates to management | Eradication report |
| Recovery | Restore operations, verify system integrity, resume monitoring | Control Eng., Operations, QA/QC | All-clear notification | Recovery verification report |
| Lessons Learned | Post-incident review, update procedures, retrain as needed | All roles, Management | Final report distribution | Lessons learned report |

# 9. Modification, Retrofit, and Decommissioning

## 9.1 Security Assessment for Modifications

| Assessment Item | Details | Responsible | Approval |
| --- | --- | --- | --- |
| Description of modification | [Description] | [Engineer] |  |
| Affected systems and zones | [Systems] | [Engineer] |  |
| Security impact analysis | [Impact] | Security Engineer |  |
| Safety impact analysis | [Impact] | Safety Engineer |  |
| Updated risk assessment required? | [Yes/No] | Security Engineer | [Approver] |
| Updated zone/conduit diagram required? | [Yes/No] | Control Engineer | [Approver] |
| Regression testing required? | [Yes/No] | QA/QC | [Approver] |
| Updated V&V required? | [Yes/No] | Security Tester | [Approver] |

## 9.2 Decommissioning Security Checklist

| Step | Action | Completed (Y/N) | Verified By | Date |
| --- | --- | --- | --- | --- |
| 1 | Remove all credentials, keys, and certificates from device | [Y/N] | [Verifier] | [Date] |
| 2 | Securely wipe all storage media (NIST 800-88 compliant) | [Y/N] | [Verifier] | [Date] |
| 3 | Remove device from network access control lists and firewall rules | [Y/N] | [Verifier] | [Date] |
| 4 | Revoke all associated user accounts and service accounts | [Y/N] | [Verifier] | [Date] |
| 5 | Update asset inventory and network diagrams | [Y/N] | [Verifier] | [Date] |
| 6 | Archive security logs and configuration backups per retention policy | [Y/N] | [Verifier] | [Date] |
| 7 | Physical destruction of storage media if required by classification | [Y/N] | [Verifier] | [Date] |
| 8 | Final security sign-off | [Y/N] | [Verifier] | [Date] |

# 10. Plan of Action and Milestones (POA&M)

Track all identified security gaps, findings, and corrective actions across the lifecycle:

| POA&M ID | Finding Source | Description | Affected System(s) | Risk Level | Corrective Action | Responsible | Target Date | Status | Completion Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [P-001] | ICSA / DCRA / V&V / Audit | [Description] | [Systems] | [H/M/L] | [Action] | [Owner] | [Date] | Open / In Progress / Closed | [Date] |

# 11. Key Performance Indicators - (ISA 84.00.09 Edition 3 Annex L Part 1)

This section references example KPIs that can be found in 2024 version 3 Annex L of ISA84.00.09 part 1. Some of these KPI can be tracked and updated on the security workbook spreadsheet and the standalone KPI html portal tool.

## L.1 Cyber Security Management System (CSMS) Performance

### L.1.1: CSMS Policy Compliance Rate

| Field | Details |
| --- | --- |
| Description | Percentage of organizational units in full compliance with CSMS policy |
| Formula | (Units compliant / Total units) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 18 of 20 business units = 90% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.1.2: Management Review Completion Rate

| Field | Details |
| --- | --- |
| Description | Percentage of scheduled management reviews completed on time |
| Formula | (Reviews completed / Scheduled reviews) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 2 of 2 annual reviews = 100% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.1.3: Cyber Security Resource Allocation Adequacy

| Field | Details |
| --- | --- |
| Description | Ratio of actual budget/staffing vs required levels |
| Formula | (Actual resources / Required resources) x 100 |
| Unit | % |
| Target | >=90% |
| Measurement Frequency | Annually |
| Worked Example | $1.2M of $1.5M required = 80% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.1.4: Cyber Security Budget Execution Rate

| Field | Details |
| --- | --- |
| Description | Percentage of approved budget expended on planned activities |
| Formula | (Actual expenditure / Approved budget) x 100 |
| Unit | % |
| Target | 85-100% |
| Measurement Frequency | Quarterly |
| Worked Example | Q3: $280K of $350K = 80% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.1.5: Roles and Responsibilities Coverage

| Field | Details |
| --- | --- |
| Description | Percentage of required security roles filled with qualified personnel |
| Formula | (Filled qualified positions / Required positions) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Semi-annually |
| Worked Example | 14 of 16 positions = 87.5% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## L.2 Risk Assessment Performance

### L.2.1: ICSA Completion Rate

| Field | Details |
| --- | --- |
| Description | Percentage of systems/zones with completed Initial Cyber Security Assessment |
| Formula | (Systems with ICSA / Total systems) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 120/120 systems (SIS,BPCS,DCS,PLC,RTU,SCADA,HMI,IoT,BAS) = 100% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.2.2: DCRA Completion Rate

| Field | Details |
| --- | --- |
| Description | Percentage of high-risk systems with completed Detailed Cyber Risk Assessment |
| Formula | (High-risk with DCRA / Total high-risk) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 35/40 high-risk (all SIS,FGS,BMS,critical BPCS) = 87.5% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.2.3: Risk Assessment Currency Rate

| Field | Details |
| --- | --- |
| Description | Percentage of assessments current within review cycle (2-3 years) |
| Formula | (Current assessments / Total required) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 38/42 within 3-year cycle = 90.5% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.2.4: Identified Risk Mitigation Rate

| Field | Details |
| --- | --- |
| Description | Percentage of risks with implemented or accepted mitigation |
| Formula | (Risks mitigated or accepted / Total identified) x 100 |
| Unit | % |
| Target | >=95% |
| Measurement Frequency | Quarterly |
| Worked Example | 180/195 risks mitigated = 92.3% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.2.5: Residual Risk Acceptance Documentation Rate

| Field | Details |
| --- | --- |
| Description | Percentage of accepted residual risks with formal documentation and sign-off |
| Formula | (Documented accepted / Total accepted) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 15/15 have signed acceptance = 100% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.2.6: HCE Coverage Rate

| Field | Details |
| --- | --- |
| Description | Percentage of identified HCEs with completed cyber security analysis |
| Formula | (HCEs analyzed / Total HCEs) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 28/30 HCEs fully analyzed = 93.3% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## L.3 Security Architecture and Design

### L.3.1: Zone and Conduit Documentation Completeness

| Field | Details |
| --- | --- |
| Description | Percentage of zones/conduits with complete documentation per 62443-3-2 |
| Formula | (Documented zones/conduits / Total) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 24/26 fully documented = 92.3% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.3.2: Security Level Target (SL-T) Achievement Rate

| Field | Details |
| --- | --- |
| Description | Percentage of zones where SL-A meets or exceeds SL-T |
| Formula | (Zones SL-A >= SL-T / Total zones with SL-T) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 10/12 zones: Zone 1 SIS SL-T=3 SL-A=3; Zone 4 IoT SL-T=2 SL-A=1 (gap) |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.3.3: Network Segmentation Verification Rate

| Field | Details |
| --- | --- |
| Description | Percentage of zone boundaries with verified segmentation controls |
| Formula | (Verified boundaries / Total boundaries) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Semi-annually |
| Worked Example | 18/18 verified by pen test or config audit = 100% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.3.4: Defense-in-Depth Layer Coverage

| Field | Details |
| --- | --- |
| Description | Percentage of defense-in-depth layers implemented per design |
| Formula | (Implemented layers / Required layers) x 100 |
| Unit | % |
| Target | >=90% |
| Measurement Frequency | Annually |
| Worked Example | 4/5 layers (perimeter,network,host,app,data): data encryption pending = 80% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.3.5: Security Architecture Review Currency

| Field | Details |
| --- | --- |
| Description | Percentage of architecture documents reviewed and current |
| Formula | (Current documents / Total documents) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 7/8 current = 87.5% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## L.4 Threat Management

### L.4.1: Threat Intelligence Integration Rate

| Field | Details |
| --- | --- |
| Description | Percentage of relevant ICS/OT threat intel sources actively monitored |
| Formula | (Active sources / Identified relevant sources) x 100 |
| Unit | % |
| Target | >=80% |
| Measurement Frequency | Quarterly |
| Worked Example | 6/8 (ICS-CERT, vendor advisories, ATT&CK ICS, ISAC) = 75% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.4.2: Threat Assessment Update Frequency

| Field | Details |
| --- | --- |
| Description | Percentage of threat assessments updated within SLA after new intel |
| Formula | (Updated within SLA / Updates required) x 100 |
| Unit | % |
| Target | >=90% |
| Measurement Frequency | Quarterly |
| Worked Example | 4/5 updated within 30-day SLA = 80% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.4.3: Attack Surface Reduction Progress

| Field | Details |
| --- | --- |
| Description | Measured reduction in attack surface (ports, services, default accounts) |
| Formula | (Items eliminated / Baseline items) x 100 |
| Unit | % |
| Target | Continuous reduction |
| Measurement Frequency | Quarterly |
| Worked Example | 85/340 baseline items eliminated = 25% reduction |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.4.4: MITRE ATT&CK for ICS Coverage Rate

| Field | Details |
| --- | --- |
| Description | Percentage of applicable ATT&CK ICS techniques with detection/prevention |
| Formula | (Techniques with controls / Total applicable) x 100 |
| Unit | % |
| Target | >=70% |
| Measurement Frequency | Semi-annually |
| Worked Example | 52/78 applicable techniques have controls = 66.7% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## L.5 Vulnerability Management

### L.5.1: Vulnerability Identification Rate

| Field | Details |
| --- | --- |
| Description | Percentage of in-scope systems with current vulnerability assessment |
| Formula | (Systems with current assessment / Total in-scope) x 100 |
| Unit | % |
| Target | >=95% |
| Measurement Frequency | Monthly |
| Worked Example | 114/120 systems assessed <90 days = 95% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.5.2: Critical Vulnerability Remediation Rate

| Field | Details |
| --- | --- |
| Description | Percentage of critical (CVSS>=9.0) vulns remediated within SLA (72h) |
| Formula | (Critical fixed within SLA / Total critical) x 100 |
| Unit | % |
| Target | >=95% |
| Measurement Frequency | Monthly |
| Worked Example | 7/8 within 72h = 87.5%; 1 legacy PLC needed compensating control |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.5.3: High Vulnerability Remediation Rate

| Field | Details |
| --- | --- |
| Description | Percentage of high (CVSS 7.0-8.9) vulns remediated within SLA (30d) |
| Formula | (High fixed within SLA / Total high) x 100 |
| Unit | % |
| Target | >=90% |
| Measurement Frequency | Monthly |
| Worked Example | 19/22 within 30 days = 86.4% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.5.4: Medium/Low Vulnerability Remediation Rate

| Field | Details |
| --- | --- |
| Description | Percentage of med/low vulns remediated within SLA (90d) |
| Formula | (Med/Low fixed within SLA / Total med/low) x 100 |
| Unit | % |
| Target | >=85% |
| Measurement Frequency | Monthly |
| Worked Example | 40/45 within 90 days = 88.9% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.5.5: Mean Time to Remediate (MTTR) - Critical

| Field | Details |
| --- | --- |
| Description | Average time from critical vulnerability identification to remediation |
| Formula | Sum(remediation times) / Count(critical vulns) |
| Unit | Hours |
| Target | <=72 hours |
| Measurement Frequency | Monthly |
| Worked Example | 8 critical vulns: 480 hours / 8 = 60 hours avg |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.5.6: Mean Time to Remediate (MTTR) - All Severities

| Field | Details |
| --- | --- |
| Description | Average time from identification to remediation across all severities |
| Formula | Sum(all remediation times) / Count(all vulns) |
| Unit | Days |
| Target | <=30 days |
| Measurement Frequency | Monthly |
| Worked Example | 75 vulns: 1,650 days / 75 = 22 days avg |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.5.7: Patch Currency Rate

| Field | Details |
| --- | --- |
| Description | Percentage of systems running supported, patched software |
| Formula | (Fully patched / Total systems) x 100 |
| Unit | % |
| Target | >=85% |
| Measurement Frequency | Monthly |
| Worked Example | 102/120 = 85%; 12 legacy on compensating controls |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.5.8: Compensating Control Coverage for Unpatched Systems

| Field | Details |
| --- | --- |
| Description | Percentage of unpatched/unsupported systems with verified compensating controls |
| Formula | (Unpatched with controls / Total unpatched) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Monthly |
| Worked Example | 16/18 unpatched have verified controls = 88.9% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## L.6 Incident Management

### L.6.1: Mean Time to Detect (MTTD)

| Field | Details |
| --- | --- |
| Description | Average time from event occurrence to detection |
| Formula | Sum(detection delays) / Count(detected events) |
| Unit | Hours |
| Target | <=24 hours |
| Measurement Frequency | Quarterly |
| Worked Example | 12 events: 180 hours / 12 = 15 hours avg |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.6.2: Mean Time to Respond (MTTR-I)

| Field | Details |
| --- | --- |
| Description | Average time from detection to initial containment |
| Formula | Sum(response delays) / Count(incidents) |
| Unit | Hours |
| Target | <=4 hours |
| Measurement Frequency | Quarterly |
| Worked Example | 6 incidents: 18 hours / 6 = 3 hours avg |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.6.3: Mean Time to Recover (MTTR-R)

| Field | Details |
| --- | --- |
| Description | Average time from containment to full recovery |
| Formula | Sum(recovery times) / Count(incidents) |
| Unit | Hours |
| Target | <=48 hours |
| Measurement Frequency | Quarterly |
| Worked Example | 6 incidents: 192 hours / 6 = 32 hours avg |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.6.4: Incident Response Plan Test Completion Rate

| Field | Details |
| --- | --- |
| Description | Percentage of scheduled IR drills and tabletop exercises completed |
| Formula | (Drills completed / Scheduled drills) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Semi-annually |
| Worked Example | 2 tabletops + 1 full drill: 2/3 = 66.7% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.6.5: Incident Recurrence Rate

| Field | Details |
| --- | --- |
| Description | Percentage of incidents that are repeats of previously addressed incidents |
| Formula | (Recurring / Total incidents) x 100 |
| Unit | % |
| Target | <=10% |
| Measurement Frequency | Quarterly |
| Worked Example | 2/15 recurrences = 13.3% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.6.6: Lessons Learned Implementation Rate

| Field | Details |
| --- | --- |
| Description | Percentage of post-mortem recommendations implemented |
| Formula | (Implemented / Total recommendations) x 100 |
| Unit | % |
| Target | >=90% |
| Measurement Frequency | Quarterly |
| Worked Example | 24/28 from 6 incident reviews = 85.7% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## L.7 Compliance and Audit

### L.7.1: Security Policy Compliance Rate

| Field | Details |
| --- | --- |
| Description | Percentage of systems/processes compliant with security policies |
| Formula | (Compliant / Total assessed) x 100 |
| Unit | % |
| Target | >=95% |
| Measurement Frequency | Quarterly |
| Worked Example | 112/120 systems compliant = 93.3% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.7.2: Audit Finding Closure Rate

| Field | Details |
| --- | --- |
| Description | Percentage of audit findings closed within remediation timeline |
| Formula | (Closed within SLA / Total findings) x 100 |
| Unit | % |
| Target | >=90% |
| Measurement Frequency | Quarterly |
| Worked Example | 28/32 closed within 90-day SLA = 87.5% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.7.3: Regulatory Compliance Achievement Rate

| Field | Details |
| --- | --- |
| Description | Percentage of applicable regulatory requirements in compliance |
| Formula | (Requirements met / Total applicable) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | NERC CIP: 42/45 = 93.3%; 3 POA&M items for CIP-007 |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.7.4: Configuration Baseline Compliance Rate

| Field | Details |
| --- | --- |
| Description | Percentage of systems matching approved config baseline (CIS/STIG) |
| Formula | (Systems on baseline / Total baselined) x 100 |
| Unit | % |
| Target | >=90% |
| Measurement Frequency | Monthly |
| Worked Example | 108/120 match baseline = 90% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.7.5: Access Control Compliance Rate

| Field | Details |
| --- | --- |
| Description | Percentage of accounts with appropriate access per least privilege |
| Formula | (Compliant accounts / Total reviewed) x 100 |
| Unit | % |
| Target | >=95% |
| Measurement Frequency | Quarterly |
| Worked Example | 82/85 = 96.5%; 3 had excessive privileges removed |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## L.8 Training and Awareness

### L.8.1: Security Training Completion Rate

| Field | Details |
| --- | --- |
| Description | Percentage of personnel completing required role-based training |
| Formula | (Completed / Required) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 43/45 = 95.6%; 2 new hires pending |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.8.2: Role-Based Training Coverage Rate

| Field | Details |
| --- | --- |
| Description | Percentage of defined roles with role-specific training developed |
| Formula | (Roles with training / Total roles) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 5/6 roles (Operator,Engineer,Admin,Manager,Safety); SI pending = 83.3% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.8.3: Security Awareness Assessment Score

| Field | Details |
| --- | --- |
| Description | Average score on cyber security awareness quizzes |
| Formula | Sum(scores) / Number(assessments) |
| Unit | % |
| Target | >=80% |
| Measurement Frequency | Annually |
| Worked Example | 45 personnel avg score = 78%; remedial training scheduled |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.8.4: Social Engineering Exercise Performance

| Field | Details |
| --- | --- |
| Description | Percentage who correctly identified/reported phishing simulation |
| Formula | (Reported / Received simulation) x 100 |
| Unit | % |
| Target | >=85% |
| Measurement Frequency | Semi-annually |
| Worked Example | 36/45 reported, 5 clicked, 4 ignored = 80% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## L.9 Supply Chain Security

### L.9.1: Vendor Security Assessment Completion Rate

| Field | Details |
| --- | --- |
| Description | Percentage of critical ICS/OT vendors with completed assessment |
| Formula | (Vendors assessed / Total critical vendors) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Annually |
| Worked Example | 10/12 critical vendors (PLC,DCS,SIS mfgs + 3 SIs) = 83.3% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.9.2: Supply Chain Risk Monitoring Coverage

| Field | Details |
| --- | --- |
| Description | Percentage of supply chain components with active risk monitoring |
| Formula | (Components monitored / Total components) x 100 |
| Unit | % |
| Target | >=80% |
| Measurement Frequency | Quarterly |
| Worked Example | 195/250 with active CVE monitoring = 78% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.9.3: Third-Party Access Compliance Rate

| Field | Details |
| --- | --- |
| Description | Percentage of third-party sessions compliant with access policy |
| Formula | (Compliant sessions / Total sessions) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Monthly |
| Worked Example | 38/42 compliant = 90.5%; 4 exceeded time limits |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.9.4: Software Bill of Materials (SBOM) Coverage

| Field | Details |
| --- | --- |
| Description | Percentage of deployed software/firmware with documented SBOM |
| Formula | (Components with SBOM / Total deployed) x 100 |
| Unit | % |
| Target | >=75% |
| Measurement Frequency | Semi-annually |
| Worked Example | 110/180 = 61.1%; improvement plan in place |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## L.10 Operational Effectiveness

### L.10.1: Backup and Recovery Test Success Rate

| Field | Details |
| --- | --- |
| Description | Percentage of systems with verified backup and tested recovery |
| Formula | (Tested systems / Systems requiring backup) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Quarterly |
| Worked Example | 90/95 = 94.7%; 5 pending maintenance window |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.10.2: Security-Related System Availability

| Field | Details |
| --- | --- |
| Description | Uptime excluding security-related downtime |
| Formula | ((Total time - security downtime) / Total time) x 100 |
| Unit | % |
| Target | >=99.5% |
| Measurement Frequency | Monthly |
| Worked Example | 720h month: 3.2h downtime = 99.56% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.10.3: Change Management Security Review Completion

| Field | Details |
| --- | --- |
| Description | Percentage of changes with security review before implementation |
| Formula | (Changes reviewed / Total changes) x 100 |
| Unit | % |
| Target | 100% |
| Measurement Frequency | Monthly |
| Worked Example | 16/18 = 88.9%; 2 emergency changes reviewed post-hoc |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.10.4: POA&M Item Closure Rate

| Field | Details |
| --- | --- |
| Description | Percentage of POA&M items closed within target date |
| Formula | (Closed on time / Total due) x 100 |
| Unit | % |
| Target | >=85% |
| Measurement Frequency | Monthly |
| Worked Example | 17/20 = 85% |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.10.5: POA&M Item Aging (Average Days Open)

| Field | Details |
| --- | --- |
| Description | Average days that open POA&M items have been open |
| Formula | Sum(days open) / Count(open items) |
| Unit | Days |
| Target | <=60 days |
| Measurement Frequency | Monthly |
| Worked Example | 12 open items: 504 days / 12 = 42 days avg |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.10.6: Security Monitoring Coverage Rate

| Field | Details |
| --- | --- |
| Description | Percentage of in-scope systems with active security monitoring |
| Formula | (Systems monitored / Total in-scope) x 100 |
| Unit | % |
| Target | >=90% |
| Measurement Frequency | Monthly |
| Worked Example | 108/120 = 90%; 12 legacy via network-level only |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

### L.10.7: Security Control Effectiveness Score

| Field | Details |
| --- | --- |
| Description | Composite score of security control effectiveness from testing |
| Formula | (Controls effective / Total tested) x 100 |
| Unit | % |
| Target | >=85% |
| Measurement Frequency | Annually |
| Worked Example | 68/78 = 87.2%; 10 require tuning |
| Current Value | [Enter calculated value] |
| Data Source | [Specify data source] |
| Owner | [Assign responsible role] |
| Notes | [Deviations, compensating factors] |

## 11.2 Annex L KPI Summary Tracking Table

Use this consolidated table to track all Annex L KPIs. Fill in the Current Value column each reporting period.

| KPI ID | Metric Name | Category | Unit | Target | Current Value | Status | Owner |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L.1.1 | CSMS Policy Compliance Rate | L.1 Cyber Security Management System (CSMS) Performance | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.1.2 | Management Review Completion Rate | L.1 Cyber Security Management System (CSMS) Performance | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.1.3 | Cyber Security Resource Allocation Adequacy | L.1 Cyber Security Management System (CSMS) Performance | % | >=90% | [Value] | [R/A/G] | [Owner] |
| L.1.4 | Cyber Security Budget Execution Rate | L.1 Cyber Security Management System (CSMS) Performance | % | 85-100% | [Value] | [R/A/G] | [Owner] |
| L.1.5 | Roles and Responsibilities Coverage | L.1 Cyber Security Management System (CSMS) Performance | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.2.1 | ICSA Completion Rate | L.2 Risk Assessment Performance | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.2.2 | DCRA Completion Rate | L.2 Risk Assessment Performance | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.2.3 | Risk Assessment Currency Rate | L.2 Risk Assessment Performance | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.2.4 | Identified Risk Mitigation Rate | L.2 Risk Assessment Performance | % | >=95% | [Value] | [R/A/G] | [Owner] |
| L.2.5 | Residual Risk Acceptance Documentation Rate | L.2 Risk Assessment Performance | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.2.6 | HCE Coverage Rate | L.2 Risk Assessment Performance | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.3.1 | Zone and Conduit Documentation Completeness | L.3 Security Architecture and Design | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.3.2 | Security Level Target (SL-T) Achievement Rate | L.3 Security Architecture and Design | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.3.3 | Network Segmentation Verification Rate | L.3 Security Architecture and Design | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.3.4 | Defense-in-Depth Layer Coverage | L.3 Security Architecture and Design | % | >=90% | [Value] | [R/A/G] | [Owner] |
| L.3.5 | Security Architecture Review Currency | L.3 Security Architecture and Design | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.4.1 | Threat Intelligence Integration Rate | L.4 Threat Management | % | >=80% | [Value] | [R/A/G] | [Owner] |
| L.4.2 | Threat Assessment Update Frequency | L.4 Threat Management | % | >=90% | [Value] | [R/A/G] | [Owner] |
| L.4.3 | Attack Surface Reduction Progress | L.4 Threat Management | % | Continuous reduction | [Value] | [R/A/G] | [Owner] |
| L.4.4 | MITRE ATT&CK for ICS Coverage Rate | L.4 Threat Management | % | >=70% | [Value] | [R/A/G] | [Owner] |
| L.5.1 | Vulnerability Identification Rate | L.5 Vulnerability Management | % | >=95% | [Value] | [R/A/G] | [Owner] |
| L.5.2 | Critical Vulnerability Remediation Rate | L.5 Vulnerability Management | % | >=95% | [Value] | [R/A/G] | [Owner] |
| L.5.3 | High Vulnerability Remediation Rate | L.5 Vulnerability Management | % | >=90% | [Value] | [R/A/G] | [Owner] |
| L.5.4 | Medium/Low Vulnerability Remediation Rate | L.5 Vulnerability Management | % | >=85% | [Value] | [R/A/G] | [Owner] |
| L.5.5 | Mean Time to Remediate (MTTR) - Critical | L.5 Vulnerability Management | Hours | <=72 hours | [Value] | [R/A/G] | [Owner] |
| L.5.6 | Mean Time to Remediate (MTTR) - All Severities | L.5 Vulnerability Management | Days | <=30 days | [Value] | [R/A/G] | [Owner] |
| L.5.7 | Patch Currency Rate | L.5 Vulnerability Management | % | >=85% | [Value] | [R/A/G] | [Owner] |
| L.5.8 | Compensating Control Coverage for Unpatched Systems | L.5 Vulnerability Management | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.6.1 | Mean Time to Detect (MTTD) | L.6 Incident Management | Hours | <=24 hours | [Value] | [R/A/G] | [Owner] |
| L.6.2 | Mean Time to Respond (MTTR-I) | L.6 Incident Management | Hours | <=4 hours | [Value] | [R/A/G] | [Owner] |
| L.6.3 | Mean Time to Recover (MTTR-R) | L.6 Incident Management | Hours | <=48 hours | [Value] | [R/A/G] | [Owner] |
| L.6.4 | Incident Response Plan Test Completion Rate | L.6 Incident Management | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.6.5 | Incident Recurrence Rate | L.6 Incident Management | % | <=10% | [Value] | [R/A/G] | [Owner] |
| L.6.6 | Lessons Learned Implementation Rate | L.6 Incident Management | % | >=90% | [Value] | [R/A/G] | [Owner] |
| L.7.1 | Security Policy Compliance Rate | L.7 Compliance and Audit | % | >=95% | [Value] | [R/A/G] | [Owner] |
| L.7.2 | Audit Finding Closure Rate | L.7 Compliance and Audit | % | >=90% | [Value] | [R/A/G] | [Owner] |
| L.7.3 | Regulatory Compliance Achievement Rate | L.7 Compliance and Audit | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.7.4 | Configuration Baseline Compliance Rate | L.7 Compliance and Audit | % | >=90% | [Value] | [R/A/G] | [Owner] |
| L.7.5 | Access Control Compliance Rate | L.7 Compliance and Audit | % | >=95% | [Value] | [R/A/G] | [Owner] |
| L.8.1 | Security Training Completion Rate | L.8 Training and Awareness | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.8.2 | Role-Based Training Coverage Rate | L.8 Training and Awareness | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.8.3 | Security Awareness Assessment Score | L.8 Training and Awareness | % | >=80% | [Value] | [R/A/G] | [Owner] |
| L.8.4 | Social Engineering Exercise Performance | L.8 Training and Awareness | % | >=85% | [Value] | [R/A/G] | [Owner] |
| L.9.1 | Vendor Security Assessment Completion Rate | L.9 Supply Chain Security | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.9.2 | Supply Chain Risk Monitoring Coverage | L.9 Supply Chain Security | % | >=80% | [Value] | [R/A/G] | [Owner] |
| L.9.3 | Third-Party Access Compliance Rate | L.9 Supply Chain Security | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.9.4 | Software Bill of Materials (SBOM) Coverage | L.9 Supply Chain Security | % | >=75% | [Value] | [R/A/G] | [Owner] |
| L.10.1 | Backup and Recovery Test Success Rate | L.10 Operational Effectiveness | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.10.2 | Security-Related System Availability | L.10 Operational Effectiveness | % | >=99.5% | [Value] | [R/A/G] | [Owner] |
| L.10.3 | Change Management Security Review Completion | L.10 Operational Effectiveness | % | 100% | [Value] | [R/A/G] | [Owner] |
| L.10.4 | POA&M Item Closure Rate | L.10 Operational Effectiveness | % | >=85% | [Value] | [R/A/G] | [Owner] |
| L.10.5 | POA&M Item Aging (Average Days Open) | L.10 Operational Effectiveness | Days | <=60 days | [Value] | [R/A/G] | [Owner] |
| L.10.6 | Security Monitoring Coverage Rate | L.10 Operational Effectiveness | % | >=90% | [Value] | [R/A/G] | [Owner] |
| L.10.7 | Security Control Effectiveness Score | L.10 Operational Effectiveness | % | >=85% | [Value] | [R/A/G] | [Owner] |

## 11.3 KPI Reporting Template

| Reporting Element | Content |
| --- | --- |
| Report Period | [Start Date - End Date] |
| Prepared By | [Name / Role] |
| Executive Summary | [Overall security posture assessment in 2-3 sentences] |
| KPI Dashboard Summary | [Link to interactive KPI dashboard or attached chart] |
| Trending Analysis | [Month-over-month or quarter-over-quarter trends] |
| Critical Findings | [Any KPIs significantly below target with root cause] |
| Recommended Actions | [Prioritized actions to improve lagging metrics] |
| Next Review Date | [Date] |

# 12. Multi-Framework Compliance Mapping

Cross-reference compliance requirements across all applicable frameworks:

| ISA 84.00.09 Phase | ISA/IEC 62443 | NIST SP 800-82 | NIST SP 800-160 | NERC CIP (if applicable) | IEC 61511 (SIS) | MITRE ATT&CK for ICS |
| --- | --- | --- | --- | --- | --- | --- |
| Initial CSA | 62443-3-2 Zone/Conduit | Section 6 - Risk Mgmt | Ch 3 - Systems Analysis | CIP-002 - BES Identification | Clause 8.2.4 | Reconnaissance, Initial Access |
| Detailed CRA | 62443-3-3 FR Assessment | Section 6.2 - Threat Analysis | Ch 3 - Threat Modeling | CIP-005 - Electronic Perimeter | Clause 8.2.4 | All Tactics |
| Requirements | 62443-3-3 SR/RE | Section 6.3 - Controls | Ch 4 - Requirements | CIP-005, CIP-007 | Clause 11.2.12 | Detection, Response |
| Design | 62443-4-1 Secure Dev | Section 6.4 - Architecture | Ch 4 - Architecture | CIP-005, CIP-010 | Clause 11.3 | N/A |
| Implementation | 62443-4-1 Implementation | Section 6.5 - Implementation | Ch 5 - Implementation | CIP-007, CIP-010 | Clause 12 | N/A |
| V&V | 62443-4-1 Testing | Section 6.6 - Assessment | Ch 5 - Verification | CIP-010 - Change Mgmt | Clause 13/15 | All Tactics (Red Team) |
| Operations | 62443-2-1 CSMS | Section 6.7 - Monitoring | Ch 6 - Operations | CIP-006, CIP-008, CIP-009 | Clause 16 | Detection, Response |
| Mod/Decom | 62443-2-1 Change Mgmt | Section 6.8 - Change Mgmt | Ch 7 - Disposal | CIP-010, CIP-011 | Clause 17 | N/A |

# Appendix A: Glossary and Abbreviations

| Term | Definition |
| --- | --- |
| BPCS | Basic Process Control System |
| CPS | Cyber-Physical System |
| CSMS | Cyber Security Management System (ISA/IEC 62443-2-1) |
| CSRS | Cyber Security Requirements Specification |
| DCRA | Detailed Cyber Risk Assessment |
| DCS | Distributed Control System |
| EMB3D | MITRE Embedded Device Threat Model |
| FGS | Fire and Gas System |
| FR | Foundational Requirement (ISA/IEC 62443) |
| HCE | High Consequence Event |
| HMI | Human Machine Interface |
| ICSA | Initial Cyber Security Assessment |
| IoT | Internet of Things |
| OT | Operational Technology |
| PLC | Programmable Logic Controller |
| RTU | Remote Terminal Unit |
| SCADA | Supervisory Control and Data Acquisition |
| SIS | Safety Instrumented System |
| SL | Security Level (SL-T: Target, SL-C: Capability, SL-A: Achieved) |
| V&V | Verification and Validation |